| Cat.7 S/FTP Duplex Installation Cable 187311 187312 |  |  |
| --- | --- | --- |
| The equip® Duplex Installation Cable is made of preeminent materials and thoroughly examined to meet or surpass industrial standards defined by the EIA / TIA, ISO and EN standard bodies. The performance is guaranteed via a comprehensive test including attenuation, return loss, NEXT (Next End Crosstalk) and FEXT (Far End Crosstalk) up to 1000MHz and the hence the equip® Installation Cable serves well in the most demanding networking environments. |  |  |
| l Structure |  |  |
| l Specification Conductor; Material; Bare Copper. Construction; 23AWG, 1/0.56 ± 0.05mm. Insulation; Material; Skin-foam-skin PE. Thickness; MIN at any point: 0.15 mm / MAX AVG:0.22 mm. Diameter; 1.330 ± 0.05mm. Colors; Blue-White/Blue, Orange-White/Orange, Green-White/Green, Brown-White/Brown. Jacket; Material; LSZH. Thickness; 0.55mm ± 0.05mm. Diameter; (7.7-16.4) ± 0.5mm. Colors; Orange (RAL2003). Marking; 187311: equip CAT.7 Duplex S/FTP LSOH 4PR 23AWG INSTALL CABLE ISO/IEC 11801 & ANSI/TIA-568-C.2 VERIFIED EN50575:2014 Fire Class B2ca. 187312: equip CAT.7 Duplex S/FTP LSOH 4PR 23AWG INSTALL CABLE ISO/IEC 11801 & ANSI/TIA-568-C.2 VERIFIED EN50575:2014 Fire Class B2ca. Reaction to; Fire Classification; B2ca,s1,d1. |
| l Electrical Properties Electrical Characteristics; Impedance(Ω) : (1-100MHz); 100±15Ω. (100-250MHz); 100±18Ω. (250-600MHz); 100±25Ω. Delay Skew (ns/100m); <25. Velocity of Propagation (%); 74. unbalanced-to-ground capacitance (pf/100m) max; 330. DC Resistance (Ω/100m) max; 9.38. DC Conductor Resistance Unbalanc (%) max; 2. |
| l Performance |  |  |
| l Ordering Information PN; EAN; Length. 187311; 4015867202449; 500m. 187312; 4015867202456; 1000m. |